Aditya University

School of Computing
Department of Computer Applications

IoT PROJECT – REVIEW-I REPORT

IoT-Based Smart Home Garage Automation System

Automatic Vehicle Detection, Garage Gate Control and Parking Status Monitoring

| Programme | Master of Computer Applications (MCA) |
| --- | --- |
| Year / Semester | 2 / III |
| Review | Review-I |
| Review Dates | 24-08-2026 to 28-08-2026 |
| Project Guide | MSRatna Kumari mam, |

Student Team

Yandrapu Jyothika – 25M11MC056

Divili RamaLakshmi – 25M11MC026

Chikkala Anjali Devi – 25M11MC018

Amalapuram Bhargavi Priya – 25M11MC003

# 1. General Information

Project Title: IoT-Based Smart Home Garage Automation System

Project Focus: Automatic Vehicle Detection, Garage Gate Control and Parking Status Monitoring

Programme: Master of Computer Applications (MCA)

Year: 2

Semester: III

Review-I Dates: 24-08-2026 to 28-08-2026

Project Guide: Ms.U .Naga Ratna Kumari mam, Mr. K. Sai Kishore

Department: Department of Computer Applications, School of Computing, Aditya University

# 2. Project Abstract

The IoT-Based Smart Home Garage Automation System is proposed to automate vehicle entry into a home garage and reduce the need for manual gate operation. The system uses an Arduino Uno as the main controller and combines two ultrasonic sensors, an IR sensor, a servo motor and status LEDs. Ultrasonic Sensor 1 (US1) detects a vehicle approaching the garage and provides distance information to the controller. When a vehicle is detected, the green LED is activated and the servo motor opens the garage gate. An IR sensor placed at the entrance confirms that the vehicle has crossed the gate. After entry is confirmed, Ultrasonic Sensor 2 (US2) monitors the garage interior and detects the vehicle inside. The controller then closes the gate, switches the green LED OFF and turns the red LED ON to indicate that the garage is occupied. This sequence provides automatic vehicle detection, gate control, entry confirmation and basic garage status indication.

The prototype is intended as a simple, affordable and practical smart-home automation solution. The project demonstrates how multiple sensors can work together with a microcontroller to perform a complete automated sequence. The current implementation includes Arduino setup, sensor interfacing, servo control, LED status control, breadboard connections and Tinkercad simulation. The present development stage is hardware prototype development and testing. Future enhancements identified in the project include mobile-based garage status, cloud monitoring, notifications, multiple parking-slot detection, RFID/Bluetooth authentication and remote gate control.

Keywords: IoT, Smart Garage, Arduino Uno, Ultrasonic Sensor, IR Sensor, Servo Motor, Vehicle Detection, Gate Automation

# 3. Introduction

Smart-home automation aims to make routine household activities more convenient by allowing devices to respond automatically to real-world conditions. Garage gate operation is one such activity where automation can reduce manual effort and provide a more convenient entry process. Instead of requiring the vehicle owner to stop and manually operate the gate, sensors can detect the vehicle and a controller can manage the gate automatically.

The proposed system uses an Arduino Uno to coordinate vehicle detection and gate operation. US1 is positioned outside the garage to detect an approaching vehicle. An IR sensor is used at the entrance to confirm that the vehicle has crossed the gate, while US2 is positioned inside the garage to detect the vehicle after entry. A servo motor performs the gate movement and green/red LEDs provide visual status information.

The project therefore demonstrates a complete sensor-to-controller-to-actuator workflow: vehicle detection, decision making, gate opening, entry confirmation, inside detection, gate closing and garage status indication. The prototype is designed to be low-cost and suitable for demonstration using a breadboard and a compact physical garage model.

# 4. Problem Statement

Traditional home garage gates generally depend on manual operation. The vehicle owner must stop and physically open or close the gate, which can be inconvenient during busy periods or bad weather. Manual operation also does not provide a clear automatic indication of whether a vehicle is approaching, has crossed the entrance, or is inside the garage.

The major problems identified are:

Manual opening and closing of the garage gate.

Lack of automatic vehicle approach detection.

No automatic confirmation that the vehicle has crossed the entrance.

Difficulty in determining whether a vehicle is inside the garage.

Possibility of leaving the gate open unnecessarily.

Lack of simple visual indication of garage occupancy.

Therefore, an automated garage management prototype is proposed to detect the vehicle at different stages, operate the gate automatically and indicate the garage status.

# 5. Project Objectives

The main objectives of the project are:

To develop an automatic garage gate system using Arduino Uno.

To detect an approaching vehicle using an ultrasonic sensor.

To automatically open the garage gate using a servo motor.

To use an IR sensor to confirm vehicle entry.

To detect the vehicle inside the garage using a second ultrasonic sensor.

To automatically close the gate after the vehicle enters.

To indicate garage status using green and red LEDs.

To develop a simple and affordable IoT-based prototype.

# 6. Proposed IoT Modules / Functionalities

## Module 1 – Vehicle Approach Detection

HC-SR04 Ultrasonic Sensor 1 detects a vehicle approaching the garage and provides distance information to Arduino.

## Module 2 – Gate Automation

An SG90/MG90S servo motor opens the gate when a vehicle is detected and closes it after the vehicle enters.

## Module 3 – Entry Confirmation

An IR sensor at the garage entrance detects the vehicle crossing and confirms that entry has occurred.

## Module 4 – Inside Vehicle Detection

HC-SR04 Ultrasonic Sensor 2 detects the vehicle inside the garage and helps determine the occupied state.

## Module 5 – Status Indication

The green LED indicates vehicle detection/gate operation, while the red LED indicates that the garage is occupied.

## Module 6 – Control Module

Arduino Uno processes the sensor inputs and controls the servo motor and LEDs according to the programmed logic.

# 7. Hardware and Software Requirements

## Hardware Requirements

| S.No. | Component | Purpose |
| --- | --- | --- |
| 1 | Arduino Uno | Main controller |
| 2 | HC-SR04 Ultrasonic Sensor | Outside and inside vehicle detection |
| 3 | IR Sensor | Entry confirmation |
| 4 | SG90/MG90S Servo Motor | Automatic gate control |
| 5 | Green LED | Vehicle/gate status indication |
| 6 | Red LED | Garage occupied indication |
| 7 | 220Ω Resistors | LED protection |
| 8 | Breadboard | Circuit connections |
| 9 | Jumper Wires | Component connections |
| 10 | 5V Power Supply / USB | Power supply |

## Software Requirements

Arduino IDE

Tinkercad Circuits

Arduino C/C++ programming

Serial Monitor for testing and debugging

# 8. Input and Expected Output

## Inputs

Distance information from US1.

Entry detection from the IR sensor.

Distance information from US2.

Arduino analyzes the sensor readings and makes decisions using predefined distance thresholds.

## Expected Output

| Condition | Gate | Green LED | Red LED |
| --- | --- | --- | --- |
| No vehicle | Closed | OFF | OFF |
| US1 detects vehicle | Opens | ON | OFF |
| IR confirms entry | Open | ON | OFF |
| US2 detects vehicle inside | Closes | OFF | ON |
| Garage occupied | Closed | OFF | ON |

# 9. Design Methodology: Algorithm, System Flow and Flowchart

## Algorithm

Start the system.

Initialize Arduino, sensors, servo motor and LEDs.

US1 continuously measures the distance of the approaching vehicle.

If a vehicle is detected, turn ON the green LED.

Open the garage gate using the servo motor.

Wait for the IR sensor to confirm that the vehicle has crossed the entrance.

After entry confirmation, allow US2 to monitor the garage interior.

When US2 detects the vehicle inside, close the garage gate.

Turn OFF the green LED.

Turn ON the red LED.

Set the garage status to Occupied.

Continue monitoring for the next vehicle.

SYSTEM FLOW:

START

↓

Initialize System

↓

US1 Detects Car

↓

Car Detected

↓

Green LED ON

↓

Gate Opens

↓

IR Sensor Detects Vehicle Crossing

↓

Entry Confirmed

↓

US2 Detects Car Inside Garage

↓

Gate Closes

↓

Green LED OFF / Red LED ON

↓

Garage Occupied

↓

Continue Monitoring

## Flowchart

1. START

↓

2. Initialize System

↓

3. US1 Detects Car

↓

4. Car Detected?

↓

5. Green LED ON

↓

6. Gate Opens

↓

7. IR Sensor Detects Vehicle Crossing

↓

8. Entry Confirmed

↓

9. US2 Detects Car Inside Garage

↓

10. Gate Closes

↓

11. Green LED OFF / Red LED ON

↓

12. Garage Occupied

↓

13. Continue Monitoring

# 10. Implementation Progress

The following modules have been completed according to the Review-I project presentation:

Arduino Uno setup

US1 ultrasonic sensor integration

US2 ultrasonic sensor integration

IR sensor integration

Servo motor control

Green LED control

Red LED control

Breadboard circuit

Tinkercad simulation

Vehicle approach detection

Automatic gate opening

Entry confirmation

Inside vehicle detection

Automatic gate closing

Garage status indication

Current Stage: Hardware prototype development and testing.

# 11. Completed Module Demonstration and Sample Output

## Step 1 – Vehicle Approaches

US1 detects the approaching vehicle. Green LED turns ON and the servo opens the gate.

## Step 2 – Vehicle Enters

The vehicle crosses the garage entrance. The IR sensor detects the crossing and Arduino confirms entry.

## Step 3 – Vehicle Reaches Inside

US2 detects the vehicle inside the garage. The servo closes the gate, the green LED turns OFF and the red LED turns ON.

# 12. Individual Team Member Contribution

| Team Member | Contribution |
| --- | --- |
| Yandrapu Jyothika | Problem identification, literature survey, project documentation and PPT preparation. |
| Amalapuram Bhargavi Priya | Hardware selection, ultrasonic sensor interfacing and servo motor connection. |
| Divili RamaLakshmi | Arduino/ESP32 programming, vehicle detection logic and automatic gate control. |
| Chikkala Anjali Devi | IoT connectivity, garage status monitoring, dashboard development and system testing. |

# 13. Challenges Identified

Correct placement and alignment of ultrasonic sensors.

Accurate distance measurement.

Reliable IR sensor detection.

Synchronizing the three sensors.

Smooth servo gate movement.

Avoiding false sensor readings.

Managing LED status correctly.

Connecting multiple components on a single breadboard.

Creating a compact physical garage model.

# 14. Proposed Solutions

| Challenge | Proposed Solution |
| --- | --- |
| Incorrect ultrasonic readings | Calibrate distance thresholds. |
| IR detection errors | Position the IR sensor correctly at the garage entrance. |
| Sensor interference | Use separate trigger/echo pins. |
| Servo movement issues | Set suitable opening and closing angles. |
| False detection | Use appropriate threshold values. |
| LED status problems | Implement clear program conditions. |
| Wiring complexity | Use common 5V/GND breadboard rails. |
| Prototype size | Use a compact cardboard/foam-board model. |

# 15. Conclusion

The IoT-Based Smart Home Garage Automation System provides a simple and low-cost approach to automating vehicle entry and garage gate operation. US1 detects an approaching vehicle and starts the gate-opening process. The IR sensor provides confirmation that the vehicle has crossed the entrance, while US2 detects the vehicle inside the garage. Arduino Uno coordinates these sensor inputs and controls the servo motor and status LEDs. After the vehicle is detected inside, the gate closes, the green LED turns OFF and the red LED turns ON to indicate an occupied garage.

The Review-I implementation demonstrates that multiple sensors and actuators can be integrated into a single automated sequence. The completed modules and Tinkercad simulation provide a foundation for further hardware testing and system integration.

# 16. Future Enhancements

Mobile application for garage status.

Cloud-based monitoring.

Notifications when the garage is occupied.

Multiple parking-slot detection.

RFID/Bluetooth-based vehicle authentication.

Remote gate control.

# 17. Review-I Status Summary

| Activity | Status |
| --- | --- |
| Project topic selection | Completed |
| Problem identification | Completed |
| Objectives | Completed |
| Hardware and software identification | Completed |
| System flow and flowchart | Completed |
| Sensor interfacing | Completed |
| Servo and LED control | Completed |
| Tinkercad simulation | Completed |
| Hardware prototype development and testing | Current Stage |

# Source Basis

This report has been rewritten and organized from the submitted Review-I PowerPoint presentation and the supplied IoT Project Review-I report template. The technical content, modules, hardware, algorithm, implementation status, demonstration sequence, contributions, challenges, solutions and future enhancements follow the project presentation.